CPU-XA Protocol	      		                                                                  3, March 2000

Get Latched I/O

->Send 8 binary bytes:    	<200> <50> <0> <0> <0> <0> <0> <250>

<-Receive  3 byte header: 	<6> <0> <6>

<-Receive 256 bytes data:	<unit 1 Data1>  <unit 1 Data2>  <unit 2 Data1>  <unit 2 Data2>  etc…

Data Explanation:

SECU 16, Type 11   	Data1 = Latched Input Bits  0-7

Data2 = Latched Relay Bits  0-7

SECU 16I, Type 12   	Data1 = Latched Input Bits  0-7

Data2 = Latched Input Bits  8-15

RLY08-XA, Type 13	Data1 = 0

Data2 = Latched Relay Bits  0-7

Get Real Time I/O

->Send 8 binary bytes:    	<42> <0> <0> <141> <181> <0> <37> <233>

<-Receive  6 byte header: 	<n> <n> <n> <n> <n> <n>

<-Receive 256 bytes data:	<unit 1 Data1>  <unit 1 Data2>  <unit 2 Data1>  <unit 2 Data2>  etc…

<-Receive 2 byte CRC	<n> <n>

Note: You may have to program a short delay between sending the command and receiving the 264 byte response.

Data Explanation:

SECU 16, Type 11   	Data1 = Real Time Input Bits  0-7

Data2 = Latched Relay Bits  0-7

SECU 16I, Type 12   	Data1 = Real Time Input Bits  0-7

Data2 = Real Time Input Bits  8-15

RLY08-XA, Type 13	Data1 = 0

Data2 = Latched Relay Bits  0-7

Get Unit Type

->Send 8 binary bytes:    	<42> <0> <0> <1> <176> <0> <148> <39>

<-Receive  6 byte header: 	<n> <n> <n> <n> <n> <n>

<-Receive 256 bytes data:	<unit 1 Type>  <unit 2 Type>  <unit 3 Type>  <unit 4 Type>  etc…

Note: only the first 128 bytes of data are valid

<-Receive 2 byte CRC	<n> <n>

Data Explanation:

SECU 16,  Type  = 11

SECU 16I, Type  = 12

RLY08-XA, Type  = 13

Get Unit Firmware Version

->Send 8 binary bytes:    	<42> <0> <0> <130> <176> <0> <246> <45>

<-Receive  6 byte header: 	<n> <n> <n> <n> <n> <n>

<-Receive 256 bytes data:	<unit 1 Version>  <unit 2 Version>  <unit 3 Version>  <unit Version>  etc…

Note: only the first 128 bytes of data are valid

<-Receive 2 byte CRC	<n> <n>

Data Explanation:

Version is the firmware revision loaded into the unit

Initiate Get Unit Parameters

->Send 8 binary bytes:    	<200> <31> <pnum> <0> <0> <0> <0> <csum>

<-Receive  3 byte header: 	<6> <0> <6>

Note: This command instructs the CPU-XA to retrieve parameter <pnum> from every unit

Data Explanation:

pnum = the parameter number to get

csum = the 8-bit sum of  200 + 31+ pnum + 0 + 0 + 0 + 0

Write Relay Output

->Send 8 binary bytes:    	<200> <51> <unit> <relaynum> <stat> <0> <0> <csum>

<-Receive  3 byte header: 	<6> <0> <6>

Note:

Data Explanation:

unit = the unit address to write to

relaynum = the relay number to set/clear (0 to 7)

stat  = the desired status (1 = on,  0 = off)

csum = the 8-bit sum of 200 + 51 + unit + relaynum + stat + 0 + 0

Get Unit Parameters

->Send 8 binary bytes:    	<42> <0> <0> <139> <180> <0> <164> <120>

<-Receive  6 byte header: 	<n> <n> <n> <n> <n> <n>

<-Receive 256 bytes data:	<unit 1 Param LSB>  <unit 1 Param MSB>  <unit 2 Param LSB>  <unit 2 Param >MSB>  etc…

Note: this data is what is stored at parameter <pnum> in each unit

<-Receive 2 byte CRC	<n> <n>

Initiate CPU-XA RESCAN

->Send 8 binary bytes:    	<200> <32> <0> <0> <0> <0> <0> <232>

<-Receive  3 byte header: 	<6> <0> <6>

Note: This command instructs the CPU-XA rescan for active units

Initiate CPU-XA RESTART

->Send 8 binary bytes:    	<200> <52> <0> <0> <0> <0> <0> <252>

<-Receive  3 byte header: 	<6> <0> <6>

Note: This command instructs the CPU-XA do a hard reset (same as power-up)

Write Unit Parameter Data

->Send 8 binary bytes:    	<200> <33> <unit> <LSB data> <pnum> <MSB data> <0> <csum>

<-Receive  3 byte header: 	<6> <0> <6>

Note: This command instructs the CPU-XA to write  parameter <pnum> to unit

Data Explanation:

unit = the unit address to write to

LSB data = the low byte of the 16 bit data

pnum = the parameter number to write

MSB data = the high byte of the 16 bit data

csum = the 8-bit sum of 200 + 33+ unit + LSB + pnum + MSB

Write CPU-XA Parameter Data

->Send 8 binary bytes:    	<200> <40> <pnum> <MSB data> <LSB data> <0> <0> <csum>

<-Receive  3 byte header: 	<6> <0> <6>

Note: This command writes a parameter <pnum> into the CPU-XA

Data Explanation:

pnum = the parameter number to write

LSB data = the low byte of the 16 bit data

MSB data = the high byte of the 16 bit data

csum = the 8-bit sum of 200 + 40 + pnum + MSB + LSB + 0

Parameter Number Explanation:

1	-			not used

2	power mode		1 = low power mode, 0 = normal mode

3	max unit		highest unit number to scan to

4	DST status		1= DST in force,  0 = DST not in force

5	DST enable		1 = check for DST, 0 = do not check for DST

6	auto touch response

0 = do not respond

1 = respond with virtual button match

0xf2, modnum, LSBbutton, MSBbutton								2 = respond with grid location (0-59)

0xf3, modnum, LSBgrid, 0

7	rescan time		auto rescan in minutes (0 = off)

8	net timeout		time in 1/10ths sec to wait for

a module to respond

9	retry counter		give up talking to a module after

this may retries.

10	security code 1	(system interface only)

11	security code 2	(system interface only)

12	security code 3	(system interface only)

13	pager config		0 = 1 pager, 15 messages,   1 = 5 pagers, 2 messages each

14	cpu-xa num		CPU-XA address (set to 0)

15	auto X10		send X10  (0 = off, 1 = on)

0xfe (RX),hc,kc when rx’d  0xfb (TX),hc,kc when tx’d

16	auto I/O			send 0xff if remote i/o status has changed (0 = off, 1 = on)

17	auto ir			send IRnumber when compare exists, (0 = off, 1 = on)

0xfd (RX) or 0xfc (TX) , IRnumber

18	send ASCII Ir		0 = off

1 = Send ASCII string on IR recognize

+T000xxx

xxx = IR number recognized

19	Leopard			0 = CPUXA, 1 = Leopard

20	Compare IR		1-1023, Max IR to compare

21	RCS Status Response	1=On, 0= Off, Show RCS Temperatures in Vars 64-

22	Clear Variables		First Var # to leave after power-up

Send  X-10

->Send 8 binary bytes:    	<200> <55> <house> <key> <repeat> <0> <0> <csum>

<-Receive  3 byte header: 	<6> <0> <6>

Note: This command instructs the CPU-XA to add the X-10 house and key codes to it’s send buffer

Note: You may have to program a short delay between sending the command and receiving the 3 byte response.

Data Explanation:

house = the X-10 house code to send  (see Below)

key = the X-10 key code to send  (see Below)

repeat=the number of times to repeat this command (use 1 ….mostly for Dim and Bright)

csum = the 8-bit sum of 200 + 55+ house + key + repeat+ 0

| X10 House Code | CPU-XA House Code |  | X10 Key Code | CPU-XA Key Code |
| --- | --- | --- | --- | --- |
| A | 0 |  | 1 | 0 |
| B | 1 |  | 2 | 1 |
| C | 2 |  | 3 | 2 |
| D | 3 |  | 4 | 3 |
| E | 4 |  | 5 | 4 |
| F | 5 |  | 6 | 5 |
| G | 6 |  | 7 | 6 |
| H | 7 |  | 8 | 7 |
| I | 8 |  | 9 | 8 |
| J | 9 |  | 10 | 9 |
| K | 10 |  | 11 | 10 |
| L | 11 |  | 12 | 11 |
| M | 12 |  | 13 | 12 |
| N | 13 |  | 14 | 13 |
| O | 14 |  | 15 | 14 |
| P | 15 |  | 16 | 15 |
|  |  |  | All Units Off | 16 |
|  |  |  | All Lights On | 17 |
|  |  |  | On | 18 |
|  |  |  | Off | 19 |
|  |  |  | Dim | 20 |
|  |  |  | Bright | 21 |
|  |  |  | All Lights Off | 22 |
|  |  |  | Extend Code | 23 |
|  |  |  | Hail Req. | 24 |
|  |  |  | Hail Ack. | 25 |
|  |  |  | Preset Dim 0 | 26 |
|  |  |  | Extend Data | 27 |
|  |  |  | Status ON | 28 |
|  |  |  | Status OFF | 29 |
|  |  |  | Status Req | 30 |
|  |  |  | Preset Dim 1 | 31 |

Get Received  X-10

->Send 8 binary bytes: 	<200> <56> <0> <0> <0> <0> <0> <csum>

<-Receive  3 byte header: 	<6> <0> <6>

<-Receive House Code:	<n> (see Send X10 Command)

<-Receive Key Code:	<n> (see Send X10 Command)

<-Receive Buffer Size:	<n> the number of X-10 commands yet in the CPU-XA’s buffer

Note: This command instructs the CPU-XA to report it’s oldest received X-10 house and key codes

Note: This command  returns  <6> <0> <6> <99> <99> <0> if no new X-10 data is available.

Set CPU-XA Real Time Clock

->Send 50 binary bytes:	<241> <61> <0> <0> <min> <10min> <hour> <10hour> <day> <10day> <month> <10month> <year> <10year> <weekday> <0> <0> <0>……. <0> <csum>

<-Receive  3 byte header: 	<6> <0> <6>

Note: This command updates the CPU-XA’s Real Time Clock

Data Explanation:

Monday,  December 01, 1997 12:05PM translates as below…..

< 241> <61> 0  0  5  0  2  1  1  0  2  1  7  9  1  0  0  0  0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0  < 15>

Send Resident IR

->Send 8 binary bytes: 	<200> <90> <LSB IRnumber> < MSB IRnumber > <0> <0> <0> <csum>

<-Receive  3 byte header: 	<6> <0> <6>

Note: This command instructs the CPU-XA send the IR command stored in IRnumber

Data Explanation:

IRnumber = the command to be assigned to the next IR received (0 to 39)

csum = the 8-bit sum of 200 + 90+ LSB IRnumber +  MSB IRnumber +0+ 0+ 0

Initiate IR Learn

->Send 8 binary bytes: 	<200> <91> < LSB IRnumber > < MSB IRnumber > <frequency> <0> <0> <csum>

<-Receive  3 byte header: 	<6> <0> <6>

Note: This command instructs the CPU-XA learn the next IR command received and store it as command IRnumber. Valid values  for IRnumber are 0 through 39

Data Explanation:

IRnumber = the command to be assigned to the next IR received (0 to 39)

frequency = the modulation frequency (KHz) to use when sending this IR command (0 to 100)

(a frequency of  0 causes the CPU-XA to use it’s own default value)

csum = the 8-bit sum of 200 + 91+ LSB IRnumber + MSB IRnumber + frequency + 0+ 0

Load Interpreter

->Send 8 binary bytes: 	<200> <53> <lo_linenumber> <hi_linenumber> <lo_data> <hi_data> <index> <csum>

<-Receive  3 byte header: 	<6> <0> <6>

Note: This command loads a 16 bit value into a specified interpreter position

Data Explanation:

lo_linenumber = LSB of the line number to load

hi_linenumber = MSB of the line number to load

lo_data = LSB of the data to load

hi_data = MSB of the data to load

index= specific position in “line number” to load

csum = the 8-bit sum of 200+ 53+ lo_linenumber+hi_linenumber+lo_data+hi_data+index

Start Interpreter

->Send 8 binary bytes:    	<200> <54> <0> <0> <0> <0> <0> <254>

<-Receive  3 byte header: 	<6> <0> <6>

Note: This command causes the internal command interpreter to begin execution

Set CPUXA Real Time Clock

->Send 50 binary bytes: 	<241> <61> <n2> <n3>…… <n47> <n48> <csum>

<-Receive  3 byte header: 	<6> <0> <6>

Note: This command loads and starts  the CPU-XA Real Time Clock.

n2  = 61; //set RTC

n3  = seconds ones digit  	0-9 (23:59:3x)

n4  = seconds tens digit		0-5 (23:59:x4)

n5  = minutes ones digit	0-9 (23:5x:34)

n6  = minutes tens digit		0-6 (23:x9:34)

n7  = hours ones digit		0-3 (2x:59:34)

n8  = hours tens digit		0-2 (x3:59:34)

n9  = day ones digit		0-9 (12/0x/99)

n10  = day tens digit		0-3 (12/x4/99)

n11  = month ones digit	0-9 (1x/04/99)

n12  = month tens digit		0-1 (x2/04/99)

n13  = year ones digit		0-9 (12/04/9x)

n14  = year tens digit		0-9 (12/04/x9)

n15 = day of the week		0-6 (Sunday-Saturday)

csum = the 8 bit sum of 241 + 61 + n2 + n3 + n4…n47 + n48

Get CPUXA Real Time Clock

->Send 8 binary bytes: 	<200> <62> <0> <0> <0> <0> <0> <6>

<-Receive  3 byte header: 	<6> <0> <6>

<- Receive year  		<n>

<- Receive month 	<n>

<- Receive month day	<n>

<- Receive hour	  	<n>

<- Receive minute  	<n>

<- Receive weekday  	<n>

Write CPUXA Variable Data

->Send 8 binary bytes:    	<200> <41> <vnum> <LSB data>  <MSB data> <0> <0> <csum>

<-Receive  3 byte header: 	<6> <0> <6>

Note: This command writes data to vnum.

Data Explanation:

vnum = the variable number to write to (0-63)

0 - 60	Regular program variables

61	Leopard Beep Counter

Leopard will beep x times then stop

62	time conversion variable

63	Leopard Screen number

(set to 0 for user loaded Bitmap)

(set to 1-24 for C-Max designed screens)

LSB data = the low byte of the 16 bit data

MSB data = the high byte of the 16 bit data

csum = the 8-bit sum of 200 + 41+ vnum + LSB  + MSB

CPU-XA Notes:

1)	The CPU-XA interface cable to the TW-523 is a special cable and should not be substituted with a normal telephone extension cord. The correct cable pinout is listed below:

| CPU-XA |  | TW-523 |
| --- | --- | --- |
| 1 | -------> | 1 |
| 2 | -------> | 2 |
| 3 | -------> | 3 |
| 4 | -------> | 4 |

2)	The following cable pinouts are for interfacing the CPU-XA to a Macintosh computer:

| Macintosh (Din 8 Male) |  |  | CPU-XA (DB-9 Male) |  |
| --- | --- | --- | --- | --- |
| Signal | Pin |  | Pin | Signal |
| HSK o | 1 | _________ | 4 | DTR |
|  |  | ¦____ | 7 | RTS |
| HSK i | 2 | _________ | 8 | CTS |
| TxD - | 3 | _________ | 3 | Tx |
| Gnd | 4 | _________ | 5 | Gnd |
| RxD + | 8 | ____¦ |  |  |
| RxD - | 5 | _________ | 2 | Rx |
| TxD + | 6 |  | 9 |  |
| GPi | 7 | _________ | 1 | DCD |
| Shield |  | _________ |  |  |

3)	To obtain a linear value from the SECU-16 and SECU-16I analog inputs, you must remove a resistor (WHICH ONE?) from the circuit board. Removing the resistor provides a linear ouput, but also disables the supervisory capability for digital input.

Send Remote IR Command

->Send 8 binary bytes:    	<200> <92> <mnum> <znum> <IRlo> <IRhi> <0> <csum>

<-Receive  3 byte header: 	<6> <0> <6>

Note: This command sends previously learned IR command Irhi-IRlo to module mnum, zone znum

Data Explanation:

mnum = the SECU16IR address 1-128

znum = the zone on the specific SECU16IR 0-15  (znum = 255 for ALL zones)

Irlo = LSB of the ir number to send

Irhi = MSB of the ir number to send

csum = the 8-bit sum of  200 + 92+ mnum + znum + IRlo + IRhi + 0

Send Leviton X-10 Preset Dim

->Send 8 binary bytes: <200> <97> <house> <key> <dim> <49> <0> <csum>
<-Receive 3 byte header: <6> <0> <6> 
Note: This command instructs the CPU-XA to add the X-10 house, key and dim codes to it’s send buffer

Data Explanation:
house = the X-10 house code to send (see Below)
key = the X-10 key code to send (see Below)
dim =the light level 0 to 63 (0 = OFF, 63 = FULL ON)
csum = the 8-bit sum of 200 + 97+ house + key + dim + 49 + 0