Leopard Communication Protocol         7/14/00a

## V2.76 changes

Initialize touch responses from the Leopard, parameter number 6:

,Write CPU-XA Parameter Data

->Send 8 binary bytes:    	<200> <40> <pnum> <MSB data> <LSB data> <0> <0> <csum>

<-Receive  3 byte header: 	<6> <0> <6>

Note: This command writes a parameter <pnum> into the CPU-XA

Data Explanation:

pnum = the parameter number to write

LSB data = the low byte of the 16 bit data

MSB data = the high byte of the 16 bit data

csum = the 8-bit sum of 200 + 40 + pnum + MSB + LSB + 0

Parameter Number Explanation:

1	-			not used

2	-			not used

3	max unit		highest unit number to scan to

4	-			not used

5	64			internal use

6	enable touch response

0 = do not respond

1 = respond with virtual button match

2 = respond with grid location (0-59)

6 Rows of 10

0 = upper left, 59 = lower right

7	rescan time		auto rescan in minutes (0 = off)

8	-			not used

9	-			not used

10	security code 1	(system interface only)

11	security code 2	(system interface only)

12	security code 3	(system interface only)

13	max slave		highest slave address to look for  (set to 0)

14	cpu-xa num		CPU-XA address (set to 0)

15	auto X10		send X10  (0 = off, 1 = on)

0xfe (RX),hc,kc when rx’d  0xfb (TX),hc,kc when tx’d

16	auto I/O			send 0xff if remote i/o status has changed (0 = off, 1 = on)

17	auto ir			send IRnumber when compare exists,

0xfd (RX) or 0xfc (TX) , IRnumber

To Change C-Max designed screens on the fly, change variable 63:

Write CPUXA Variable Data

->Send 8 binary bytes:    	<200> <41> <vnum> <LSB data>  <MSB data> <0> <0> <csum>

<-Receive  3 byte header: 	<6> <0> <6>

Note: This command writes data to vnum.

Data Explanation:

vnum = the variable number to write to (0-63)

0 - 62	Regular program variables

63	Leopard Screen number

(set to 0 to display user loaded Bitmap)

(set to 1-24 for C-Max designed screens)

LSB data = the low byte of the 16 bit data

MSB data = the high byte of the 16 bit data

csum = the 8-bit sum of 200 + 41+ vnum + LSB  + MSB

To Write ASCII Text to the Touch Screen:

# Write Text to Leopard Touch Screen

->Send 50 binary bytes:

<241> <109> <attributes> <count> <LSB xcoord data> <MSB xcoord data>

<LSB ycoord data> <MSB ycoord data> <ASCII Char 0> …<ASCII Char  n-1>

<Pad with 0’s> <50th byte csum>

<-Receive  3 byte header: 	<6> <0> <6>

Note: This command writes ASCII chars to the Leopard Touch Screen.

Data Explanation:

attributes  =  bit numbers: 76543210

0 :  1 = turn on backlight / 0 = turn off backlight   v2.76 ..

1:   0 = use 8X12 Font

1 = use 12X16 Font

2:   1 = clear screen

3:   1 = beep once

4:   future

5:   future

6:   future

7:   future

count  =  the length of the text string to write (1-39)

LSB xcoord data = the low byte of the upper left X pixel location to write text

MSB xcoord data = the high byte of the upper left X pixel location to write text

LSB ycoord data = the low byte of the upper left Y pixel location to write text

MSB ycoord data = the high byte of the upper left Y pixel location to write text

(note: X coords 0-319 go left to right, Y coords  0-239 go top to bottom)

(note: X coords are on a 8 pixel grid)

<ASCII Char 0> …<ASCII Char  n-1> = Text to write

csum = the sum of  the 49 preceding bytes